ASSIGNMENT 2 | VERSION 4

Obstacle Avoidance: Bug Algorithm Scenario 2

Current instruction and proposed clearer student-facing revision

Review status: Draft only. No assignment configuration, grader, starter code, image, or application behavior has been changed.

# Part 1 - Current instruction

Source: backend/assignments/assignment_2.json, version 4. Formatting is normalized for review; wording is unchanged.

Scenario

A robot travels east along the Bug2 m-line from O(0,0) toward G(10,0). A rectangle blocks the m-line, with corners A(4,-1), B(6,-1), C(6,1), and D(4,1).

Specified Bug2 route

The robot reaches the obstacle at hit point H(4,0). A right turn from its eastward heading sends it toward the lower boundary. It follows H -> A -> B -> L, where the leave point is L(6,0). Point L is back on the m-line and is closer to the goal than H, so the robot leaves the boundary and continues straight to G.

Your task

- Calculate distance_to_obstacle from O to H.

- Calculate distance_along_obstacle as the sum of H-to-A, A-to-B, and B-to-L. Do not use the full rectangle perimeter.

- Calculate distance_from_obstacle_to_goal from L to G.

- Set total_path_length to the sum of those three traveled components.

Use Euclidean distance for each segment. The robot turns at exact points and its size and turning radius are ignored.

# Part 2 - Proposed improved instruction

## Purpose

Translate a specified Bug2 detour into distance calculations without using the full rectangle perimeter.

## Scenario and fixed route

A robot starts at O = (0, 0) and travels east toward G = (10, 0) along the Bug2 m-line. A rectangular obstacle has corners A = (4, -1), B = (6, -1), C = (6, 1), and D = (4, 1).

- Hit point: H = (4, 0).

- Leave point: L = (6, 0).

- Required right-turn route: O -> H -> A -> B -> L -> G.

L is on the m-line and is closer to G than H, so the robot leaves the boundary at L.

Specified lower-boundary Bug2 route for Assignment 2.

## Your task

- Set distance_to_obstacle to the Euclidean distance from O to H.

- Set distance_along_obstacle to H-to-A plus A-to-B plus B-to-L.

- Set distance_from_obstacle_to_goal to the Euclidean distance from L to G.

- Set total_path_length to the sum of those three traveled components exactly once.

distance = sqrt((x2 - x1)^2 + (y2 - y1)^2)

## Rules and required outputs

- Follow exactly H -> A -> B -> L. Do not use C, D, the upper boundary, or the full perimeter.

- Treat the robot as a point and ignore its width and turning radius.

- Required numeric variables: distance_to_obstacle, distance_along_obstacle, distance_from_obstacle_to_goal, and total_path_length.

## Before you run the tests

- Confirm that the boundary distance contains exactly three segments.

- Confirm that total_path_length contains the approach, boundary, and departure components once each.

## What this revision improves

- Uses one unambiguous coordinate layout and route direction.

- Clearly distinguishes the traveled lower boundary from the full perimeter.

- Names every graded variable and its intended path segment.

Decision requested: Approve, revise, or reject this proposed wording before any application implementation.