Question: Improving GPS Accuracy with Differential GPS (DGPS) for an Autonomous Car

Scenario:

An autonomous car is navigating a highway using GPS for positioning. However, GPS alone has an error margin of ±5 meters, which is unacceptable for lane-keeping and precise navigation. To improve accuracy, the car uses a Differential GPS (DGPS) system, which corrects GPS errors using a ground-based reference station.

The car’s true position is calculated using the correction factor from the DGPS reference station. The corrected position is given by:

Corrected Position=GPS Position+DGPS Correction

where the DGPS correction is obtained from a nearby reference station, which has an accurately known position and can measure GPS errors in real-time.

Task:

Write a Python program that:

- Takes as inputs:

  - Raw GPS position of the car (latitude, longitude)

  - DGPS reference station position (latitude, longitude)

  - Measured GPS position of the reference station (simulated GPS error)

- Computes the GPS error as:

GPS Error=Measured GPS Position−True Reference Position

- Applies this correction factor to the car’s GPS position.

- Outputs the corrected position of the car.

Hints:

- Assume small distances, so latitude/longitude errors can be added directly.

- The correction factor from the DGPS reference station improves GPS accuracy from ±5 meters to ±0.5 meters.

- Use simple subtraction and addition to apply the correction.