Question: Autonomous Underwater Vehicle (AUV) Navigation in GPS-Denied Environments

Scenario:

An Autonomous Underwater Vehicle (AUV) is deployed for deep-sea exploration where GPS signals do not penetrate underwater. The AUV relies on a Hybrid Navigation System combining:

- Inertial Navigation System (INS) → Provides position updates using accelerometers & gyroscopes, but drifts over time.

- Acoustic Navigation (SONAR + DGPS) → Uses underwater beacons to correct INS drift.

- Surface GPS Readings → When the AUV surfaces, it gets an accurate GPS fix before diving again.

The AUV operates in three phases:

- Surface Phase: Uses GPS + INS for accurate positioning.

- Underwater Phase: Uses INS + SONAR corrections due to lack of GPS.

- Resurfacing: AUV corrects drifted position using GPS and continues.

Navigation Error Model:

- INS Error Growth Rate: 0.05% of distance traveled (INS drifts over time).

- SONAR Correction Factor: Reduces INS error by 50% every 1000 meters.

- GPS Resurfacing Correction: Resets error to zero when the AUV surfaces.

Task:

Write a Python program that:

- Takes user input for:

  - Total mission distance (meters).

  - Max dive distance before resurfacing (meters).

- Simulates AUV navigation over multiple dive-resurface cycles:

  - Uses INS + SONAR while underwater.

  - Applies GPS correction when resurfacing.

  - Tracks position error over time.

- Outputs:

  - The final position error after the mission.

  - A log of how the error grew and was corrected at each resurfacing.

Hints:

- Loop through dive-resurface cycles based on the total mission distance.

- Accumulate INS error over time, then apply SONAR correction & GPS resets.

- Track and print the position error after each phase.