Question: Optimizing Wheel Configuration for a Warehouse Robot

Scenario:

A logistics company is deploying an autonomous warehouse robot to transport goods across a large distribution center. The robot needs to navigate efficiently, avoiding obstacles while ensuring stability and maneuverability.

The company is considering three different wheeled locomotion systems:

- Differential Drive (Two independently driven wheels)

- Ackermann Steering (Car-like steering mechanism)

- Omnidirectional Drive (Four Swedish wheels for full maneuverability)

The warehouse environment consists of:

- Straight aisles (2m wide)

- Sharp 90-degree turns at intersections

- Randomly placed obstacles (requiring path adjustments)

- Maximum speed limit: 1.5 m/s

Task:

Write a Python program that simulates the robot’s navigation using different wheeled locomotion models. The program should:

- Model the movement behavior of each wheeled locomotion system.

- Calculate the total time taken for the robot to navigate a 10m x 10m warehouse section with one turn and one obstacle avoidance event.

- Compare the efficiency of each system based on time taken and path adjustments.

Hints:

- Assume differential drive takes extra time during turns due to in-place rotation.

- Ackermann steering requires a turning radius, meaning it cannot turn instantly.

- Omnidirectional drive allows sideways movement, reducing turn time.

- Use a simple time = distance / speed model but add penalties for turning and obstacle avoidance.