Question: Stability Check for a Four-Legged Robot

Scenario:

A quadruped (four-legged) robot is designed for rough terrain exploration. To maintain static stability, its Center of Gravity (CoG) must stay inside the region formed by its four feet at all times.

The robot has four legs positioned at:

- Front-left leg: (1,1)

- Front-right leg: (4,1)

- Back-right leg: (4,4)

- Back-left leg: (1,4)

The robot’s CoG (x, y) is provided by its sensors.

Task:

Write a Python program that:

- Takes the four leg positions as input.

- Takes the robot’s CoG (x, y) as input.

- Checks if the CoG is inside the rectangular region formed by the four feet.

- Prints "Stable" if the CoG is inside, otherwise prints "Unstable".

Hints:

- The robot is stable if the CoG’s x-coordinate lies between the leftmost and rightmost legs AND the y-coordinate lies between the front and back legs.

- This simplifies to:

  - min_x ≤ CoG_x ≤ max_x

  - min_y ≤ CoG_y ≤ max_y

- Use basic min() and max() functions to find the boundaries of the feet positions.