Mathematical equations

In this section we will use physics concepts and mathematical formulas for calculations

- Linear velocity - the displacement of the body over time

  - Linear velocity = average of wheel speeds * wheel radius

  - Linear velocity = distance per revolution * rotational speed

  - Linear velocity = angular velocity * radius

  - Linear velocity = distance / time

- Angular velocity - refers to how fast a rigid body rotates with respect to its center of rotation or axis.

  - Angular velocity = (right wheel speed - left wheel speed) / distance between wheels

  - Angular velocity = (π * diameter* rpm) / distance between wheels

  - Angular velocity = linear velocity / radius

  - Angular velocity = angle turned / time taken.

- Revolutions per minute - measure of frequency of rotation. It states the number of full rotations per minute around a fixed axis

- Maximum rotational speed - the maximum number of revolutions a rotating system makes within a defined period of time

- Maximum linear velocity - the maximum rate of change of linear displacement with respect to time.

Other concepts and mathematical formulas

- Degrees of freedom - defines the motion capabilities; number of joints or axes of motion on an object or robot

- Cartesian degrees of freedom - the way objects are able to move along the three axes of a cartesian plane (X,Y, and Z)

- Circumference = π * diameter

- Distance traveled per rotation = Circumference

- Degrees per distance = 360 degrees ÷ Distance traveled per rotation

- Wheel rotation speed in radians = wheel rotation speed in RPM *  0.10472 rad/s

- Distance covered = Linear velocity * Time

- Number of rotations = distance covered / circumference

Math Problems

Question 1:

What are the Cartesian DoFs of a push lawnmower with four wheels? How is it still possible to mow an entire lawn with one, even though the wheels don’t yaw?

Answer:

- Translation in the x-direction (forward/backward movement)

- Translation in the y-direction (sideways movement)

- Rotation about the z-axis (yaw or turning)

Even though the wheels of a push lawnmower do not yaw, it is still possible to mow an entire lawn because the rotation of the blades provides the necessary cutting action while the operator moves the mower in different directions to cover the entire area.

Question 2:

Is a car statically or dynamically stable? What about a motorcycle?

Answer:

A car is statically stable, meaning it maintains its stability based on its design and weight distribution when it's at rest. A motorcycle, on the other hand, is dynamically stable. It requires active control and balance from the rider to maintain stability, especially during turns and maneuvers.

Question 3:

What are the Cartesian DoFs of an office chair with all caster-wheels?

Answer:

An office chair with caster wheels has three Cartesian Degrees of Freedom:

Translation in the x-direction (forward/backward movement)

Translation in the y-direction (sideways movement)

Rotation about the z-axis (yaw or spinning)

Question 4:

What are the maximum Cartesian DoFs for orientable objects driving on the 2D plane?

Answer:

For orientable objects driving on the 2D plane, the maximum Cartesian Degrees of Freedom are three:

Translation in the x-direction (forward/backward movement)

Translation in the y-direction (sideways movement)

Rotation about the z-axis (yaw or turning)

Question 5:

What are the maximum Cartesian DoFs for objects that can freely translate and rotate in the world?

Answer:

For objects that can freely translate and rotate in the world, the maximum Cartesian Degrees of Freedom are six:

Translation in the x-direction (forward/backward movement)

Translation in the y-direction (sideways movement)

Translation in the z-direction (up/down movement)

Rotation about the x-axis (roll)

Rotation about the y-axis (pitch)

Rotation about the z-axis (yaw)

Question 6:

Calculate the Cartesian DoFs of a differential drive robot with two powered rear wheels and a central, front-mounted caster wheel. What happens when you add a second caster wheel?

Answer:

For a differential drive robot with two powered rear wheels and a central,

front-mounted caster wheel:

The Cartesian Degrees of Freedom include:

Translation in the x-direction (forward/backward movement)

Translation in the y-direction (sideways movement)

Rotation about the z-axis (yaw or turning)

When you add a second caster wheel, it increases the stability of the robot but doesn't add any additional Cartesian DoFs.

Question 7:

Calculate the Cartesian DoFs of a standard car. How is it possible to still reach every point on the plane?

Answer:

The Cartesian Degrees of Freedom include:

Translation in the x-direction (forward/backward movement)

Translation in the y-direction (sideways movement)

Rotation about the z-axis (yaw or turning)

A car can reach every point on the plane by combining translation and rotation movements.

Question 8:

A steering wheel allows you to change the yaw of your car. Can you also change its pitch and its roll? See Figure 2.3 for reference.

Answer:

A steering wheel allows you to change the yaw of your car, but it does not directly control the pitch and roll. The pitch and roll of a car are primarily controlled by the suspension system and the distribution of weight during acceleration, deceleration, and turning

Question 9:

You have a robot, a differential drive robot with standard wheels. The wheels have a diameter of 12cm and are 20 cm apart. Suppose that the left wheel of the robot is rotating at 13 rpm and the right wheel is rotating at 10 rpm per minute.

  - Calculate the circumference of the wheels and find how many centimeters does the wheel travel per 360 degree rotation

  - In order to travel 20 cm, how many degrees does the wheel need to turn?

  - What is the linear velocity of the robot?

  - What is the angular velocity of the robot?

  - After 5 minutes, how much distance does the robot cover?

  - What is the turn rate of the robot after 5 minutes?

Answer:

Given:

- Diameter of the wheels: 12 cm

- Distance between the wheels: 20 cm

- Left wheel rotation: 13 rpm

- Right wheel rotation: 10 rpm

- Calculate the circumference of the wheels and find how many centimeters does the wheel travel per 360 degree rotation

Circumference = π * diameter

Circumference = 3.14159 * 12 cm

Circumference ≈ 37.6991 cm

To find how many centimeters the wheel travels per 360-degree rotation:

Distance traveled per rotation = Circumference

Distance traveled per rotation ≈ 37.6991 cm

- In order to travel 20 cm, how many degrees does the wheel need to turn?

To travel 20 cm, we need to calculate how many degrees the wheel needs to turn:

Degrees per distance = 360 degrees / Distance traveled per rotation

Degrees per distance = 360 degrees / 37.6991 cm

Degrees per distance ≈ 9.5297 degrees/cm

- What is the linear velocity of the robot?

To calculate the linear velocity we first need to convert the wheel rotation speeds to radians with: Wheel rotation speed  in radians = wheel rotation speed *  0.10472 rad/s

For left wheel:

= 13  *  0.10472 rad/s = 1.36136 rad/s

For right wheel:

= 10 *  0.10472 rad/s = 1.0472 rad/s

Now to calculate the linear velocity:

Linear velocity = average of wheel speeds * wheel radius

Wheel radius is 6 cm

(( 1.36136 + 1.047) / 2) * 6 cm  ≈ 7.22508 cm/s

- What is the angular velocity of the robot?

To calculate the angular velocity of the robot we use the wheel rotation speeds in radians from the previous exercise:

left wheel = 1.36136 rad/s

right wheel = 1.0472 rad/s

We will use the formula:

Angular velocity = ( right wheel speed - left wheel speed ) / distance between wheels

( 1.047  - 1.36136 ) / 20 cm ≈ -0.015718 cm/s

- After 5 minutes, how much distance does the robot cover?

After 5 minutes, the distance the robot covers can be calculated using the linear velocity:

Distance covered = Linear velocity * Time

Distance covered =7.22508 cm  * 5 min

Distance covered ≈ 36.1254 cm/min

- What is the turn rate of the robot after 5 minutes?

Turn rate = Angular velocity * Time

Turn rate = -0.015718 cm * 5 min

Turn rate ≈ -0.07859 cm/min

Question 10:

Say you have a differential drive robot that has an axle length of 30 cm (distance between wheels) and a wheel diameter of 10 cm. Find the angular velocity for the left and right wheel if the robot is going to:

- Spin in place at a rate of 6 rpm

- Drive a circle of radius 1 meter (1 meter = 100 cm)

- Drive a straight line at 1 meter/min

Answer:

- Spin in place at a rate of 6 rpm

Calculate the angular velocity (w)  using the formula:

w = (π * d * rpm) / distance between wheels

Given:

Wheel diameter (d) = 10 cm

Desired rate of rotation per minute (rpm) = 6 rpm

Angular distance = (π * 10 cm * 6 rpm) / (30 cm) = 6.28 rad/min

- Drive a circle of radius 1 meter (1 meter = 100 cm)

Determine the linear velocity (v) for driving at 1 m/min:

Linear velocity (v) = 1 m/min = 100 cm/min

Calculate the angular velocity using the formula: w = v / (r * 2π)

Radius of the circle (r) = 100 cm

w  = 100 cm/min / (100 cm * 2π) = 0.1592 rad/min

- Drive a straight line at 1 meter/min

Determine the linear velocity (v) for driving at 1 m/min:

Linear velocity (v) = 1 m/min = 100 cm/min

Calculate the angular velocity using the formula: w = v / (r * 2π)

In the case of driving in a straight line, the radius (r) can be considered infinitely large or approaching infinity.

w = 100 cm/min / (∞ * 2π) = 0 rad/min

For driving in a straight line, the angular velocity for both wheels is zero since there is no rotation involved.

Question 11:

Suppose you have a robot that goes at a linear velocity of 10.0213 cm/s. You know that the robots’ wheels rotate at the same speed and their radius is 4 cm.

- At what speed do the wheels rotate?

- How many rotations does the robots’ wheels need to take in order to travel 30 cm

- At what speed do the wheels need to rotate so that the robot is facing in the opposite direction in 5 mins?

Answer:

- At what speed do the wheels rotate, in radians?

Given:

Linear velocity of the robot is (v) = 10.0213 cm/s

Radius of the wheels (r) = 4 cm

The linear velocity of a robot is the same as the linear velocity of the wheels since they rotate at the same speed. To calculate the speed of each wheel we need to find the angular velocity (w).

The formula for linear velocity is: v = w * r

We rearrange it and we get: w = v / r

Now to calculate we just need to substitute in the formula and calculate:

w  = 10.0213 cm/s / 4 cm ≈ 2.5053 rad/s

- How many rotations does the robots’ wheels need to take in order to travel 30 cm?

To calculate the number of rotations required we need to divide the distance traveled by the circumference of the wheel. Let’s denote number of rotations as (N) and the formula is: N = distance / circumference

Calculate the circumference:

Circumference = π * 8 cm

Circumference of the wheels is ≈  25.1327 cm.

Now let’s calculate the number of rotations required by substituting on the formula;

N = 30 cm / 25.1327 cm

N  ≈ 1.1946

- At what speed do the wheels need to rotate so that the robot is facing in the opposite direction in 5 mins?

To calculate, we need to find the angular velocity(w) required to make a complete 180 degree turn in 5 min

Given:

Angle turned is 180 degrees

Time taken is ( 5 min * 60 seconds / minute )

Formula: w = angle turned / time taken.

By substituting in the formula we get:

w = 180 degrees / ( 5 min * 60 seconds / minute )

w ≈ 0.6 degrees per second

However, since the wheels rotate at the same speed, each wheel needs to make half of the required angular velocity, i.e 0.3 degrees per second.

Now we can convert the angular velocity to the speed of the wheel rotation:

Formula: v = w * r

Substituting the values we have:

v = 0.3 degrees/second * 4 cm ≈ 1.2 cm/seconds

Therefore, the wheels need to rotate at a speed of ≈ 1.2 cm/second to make the robot face the opposite direction in 5 minutes

Question 12:

A robot with an omni-directional drive system has four wheels arranged in a square. Each wheel has a diameter of 5 centimeters.  Calculate the maximum linear velocity of the robot when all wheels are moving at their maximum rotational speed of 10 radians per second. Additional source: Omnidirectional wheeled robot example

Answer:

To calculate the maximum linear velocity of the robot, we need to determine the distance covered by a wheel in one second when rotating at its maximum speed:

We need to calculate the circumference of the wheels with the previous formula:

Circumference = π * diameter

Circumference = π * 5 cm

Circumference = 15.7 cm

Since the wheels are arranged in a square configuration, the distance traveled by the robot for one complete revolution of a wheel would be equal to the distance covered along the diagonal of the square. We can calculate the diagonal distance (d) with the pythagorean theorem. Since the wheels are arranged in a square, the side length would be equal to the diameter of the wheel, which is 5 cm.

Diagonal distance(d) = side2+ side2

Calculate the diagonal distance (d) with the pythagorean theorem:

d  =  5cm2+ 5cm2=  50cm  ≈ 7.07cm

Find the linear velocity, by multiplying the distance traveled per revolution by the rotational speed of the wheel.

Linear velocity = distance per revolution * rotational speed

Linear velocity = 7.07 cm * 10 radians/second ≈ 70.7 cm/second

Therefore, the maximum linear velocity of the robot would be approximately 70.7 cm/second when all wheels are moving at their maximum rotational speed of 10 radians per second.

Extra Question:

A differential drive robot has wheels that are 30 cm apart. Each wheel has a diameter of 10 cm. The right wheel starts rotating at 10 RPM and the left wheel at 15 RPM. After every minute, the right wheel increases its speed by 2 RPM and the left wheel decreases its speed by 1 RPM.

- What is the angular velocity of the robot in rad/s and the linear velocity of the robot in cm/s after 3 minutes?

- At what time will the robot start rotating in the opposite direction?

- If the robot continues this pattern indefinitely, what is the average angular velocity and linear velocity of the robot over the first 10 minutes?

Hint: The angular velocity can be calculated as the difference in wheel speeds divided by the distance between the wheels. The linear velocity can be calculated as the average of the wheel speeds times the wheel radius. Remember to convert RPM to rad/s when calculating angular velocity. 1 RPM = 0.10472 rad/s.

Solution:

Angular and Linear Velocity After 3 Minutes:

After 3 minutes, the right wheel is rotating at 10 RPM + (3*2) RPM = 16 RPM,

and the left wheel is rotating at 15 RPM – (3*1) RPM = 12 RPM.

Converting these to rad/s (1 RPM = 0.10472 rad/s) we get,

for the right wheel = 16*0.10472 = 1.67552 rad/s, and

for the left wheel =12*0.10472 = 1.25664 rad/s

The angular velocity is

= (right wheel speed - left wheel speed) / distance between wheels

= (1.67552 - 1.25664) / 30

= 0.01396 rad/s.

The linear velocity is

= the average of the wheel speeds times the wheel radius

= ((1.67552 + 1.25664) / 2) * 5

= 7.331 cm/s.

Time When Robot Starts Rotating in the Opposite Direction

The robot will start rotating in the opposite direction when the right wheel speed is less than the left wheel speed.

This happens when 10 RPM + 2*t = 15 RPM - t, where t is the time in minutes.

Solving this equation gives t = 5/3 minutes.

So, the robot will start rotating in the opposite direction after 5/3 minutes, or 1 minute and 40 seconds.

Average Angular and Linear Velocity Over the First 10 Minutes

The average angular velocity is the integral of the angular velocity over the first 10 minutes divided by 10 minutes.

The angular velocity at time t is (10 RPM + 2t - (15 RPM - t)) / 30 converted to rad/s.

Integrating this from 0 to 10 and dividing by 10 gives an average angular velocity of 0.01745 rad/s.

Similarly, the average linear velocity is the integral of the linear velocity over the first 10 minutes divided by 10 minutes.

The linear velocity at time t is ((10 RPM + 2t + 15 RPM - t) / 2) * 5 converted to cm/s.

Integrating this from 0 to 10 and dividing by 10 gives an average linear velocity of 7.854 cm/s.

References:

Introduction to Mobile Robotics : Wheels and Distance Investigation

Correll, N., Hayes, B., Heckman, C., & Roncone, A. (2022). Introduction to Autonomous Robots: Mechanisms, Sensors, Actuators, and Algorithms. Mit Press.

Rotation of an Object About a Fixed Axis