Autonomous Robot Navigation in a Dynamic Office Environment

Scenario:

An autonomous delivery robot equipped with SLAM technology navigates a recently renovated multi-story office building. Starting from position S(5,5) on the 5th floor, the robot's task is to deliver documents to a goal location G(25,30) on the 10th floor. The building features two elevators: a main elevator Em​ at (10,5) and a service elevator Es​ at (6,15), with different speeds and wait times due to their usage patterns.

During its delivery task, the robot encounters an unexpected obstacle: a temporary construction barrier that extends linearly from (8,5) to (8,10). Moreover, as the robot approaches the elevators, it detects a movable obstacle blocking its most direct path.

Given Data:

- Main Elevator (Em​) Speed: 1 floor/15 seconds, Wait Time: 30 seconds (crowding)

- Service Elevator (Es​) Speed: 1 floor/20 seconds, Wait Time: 10 seconds

- Robot Speed: 1 meter/second

- Obstacle Detection Range: 2 meters

- Safety Margin: 0.5 meters from obstacles

Task:

Calculate the optimal path for the robot to reach goal G from its starting position S, considering the elevators' locations, speeds, wait times, and the obstacles encountered. The robot must update its route dynamically upon detecting the temporary barrier and the movable obstacle.

Questions:

- Path to Elevators: Calculate the shortest path lengths from S to both Em​ and Es​, taking into account the temporary construction barrier. Consider the robot's ability to detect obstacles and the required safety margin.

- Elevator Choice and Total Time to Goal: Based on the calculated paths and elevator characteristics, decide which elevator (Em​ or Es​) the robot should use. Compute the total time to reach the goal G on the 10th floor using the chosen elevator, including navigation and elevator wait and travel times.

- Dynamic Obstacle Detour: If the robot detects a movable obstacle 2 meters ahead on its path to the chosen elevator, requiring a 1-meter detour radius, recalculate the adjusted path length to the elevator. How does this affect the robot's total time to the goal?

- Comprehensive Solution: Integrate the calculations to provide a complete solution that includes the path planning to the elevator, the selection of the elevator based on time efficiency, and adjustments for dynamic obstacles, ensuring the safety margin is maintained.