Scenario

A mobile robot is equipped with a range sensor that provides distance readings within a 180-degree field of view. The sensor's view is divided into 10 equal angular sectors, each spanning 18 degrees. During its current scan, the robot detects obstacles at the following distances and angles:

- Obstacle 1: 3 meters at 30 degrees

- Obstacle 2: 5 meters at 100 degrees

- Obstacle 3: 2 meters at 150 degrees

For navigation, the robot uses a polar histogram representation. The certainty of an obstacle's presence within a given sector decreases linearly with distance. To be considered 'blocking', the certainty value of an obstacle within a sector must exceed 0.6.

Questions

- Normalization: To make the polar histogram useful for decision-making, the certainty values in each sector need to be normalized.

  - Assuming the maximum possible obstacle detection range is 6 meters, calculate the normalized certainty value for each of the three detected obstacles.

  - Hint: Obstacle certainty should be highest when it's closest.

- Blocked Sectors: Using the normalized certainty values calculated in part 1, determine which sectors within the polar histogram are considered 'blocked' (i.e., have a certainty value greater than 0.6).

Solution Outline (Instructor Notes)

- Normalization: Students would need to apply a linear function to convert distance within a sector to certainty, with 0 distance = 1.0 certainty and 6 meters = 0.0 certainties. Then, values are scaled so they sum to 1 across all sectors.

- Blocking Threshold: A simple comparison will identify the sectors having normalized values above 0.6.

Solution

Step 1: Certainty Calculation

We need a linear function to map distance to certainty within each sector. Let 'd' be the distance of the obstacle:

- certainty = (6 - d) / 6

  - At 0 meters, certainty = 1

  - At 6 meters, certainty = 0

Calculating certainties:

- Obstacle 1: (6 - 3) / 6 = 0.5

- Obstacle 2: (6 - 5) / 6 = 0.1667

- Obstacle 3: (6 - 2) / 6 = 0.6667

Step 2: Normalization

- Sum the raw certainties: 0.5 + 0.1667 + 0.6667 = 1.3334

- Divide each certainty by the sum to normalize:

  - Obstacle 1: 0.5 / 1.3334 = 0.375

  - Obstacle 2: 0.1667 / 1.3334 = 0.125

  - Obstacle 3: 0.6667 / 1.3334 = 0.5

Step 3: Identifying Blocked Sectors

Sectors with a normalized certainty value exceeding 0.6 are blocked. Therefore:

- Obstacle 1 (0.375) doesn't block its sector.

- Obstacle 2 (0.125) doesn't block its sector.

- Obstacle 3 (0.5) doesn't block its sector.

Based on the sensor readings and calculations, none of the sectors within the polar histogram are considered blocked according to the defined threshold.